# Global Electric Bus Market: Fleet Size, Manufacturers, and Technology Evolution

David Leitch

2026-03-16

# Summary

The global electric bus fleet reached approximately 780,000 vehicles at end-2024, with over 70,000 sold during the year (International Energy Agency, 2025). China dominates overwhelmingly — its 680,000+ electric buses represent roughly 87% of the global stock, and 30% of all buses on Chinese roads are now electric. Outside China, Europe is the fastest-growing market, with EU city bus registrations hitting 60% zero-emission in 2025 (Sustainable Bus, 2026c). India overtook the United States in sales volume in 2024.

Battery technology has progressed dramatically: typical 12m bus battery capacity has grown from 100–340 kWh in 2010–2014 to 400–740 kWh in current models, while pack costs have fallen roughly 90% since 2008 (Teslarati, 2024).

# Global Fleet and Sales

## Fleet stock

The IEA’s Global EV Outlook 2025 reports approximately 780,000 electric buses on the world’s roads at end-2024 (International Energy Agency, 2025). China accounts for 680,000+ of these, followed by Europe, India, and the United States.

Global Electric Bus Fleet Stock (end-2024)

| Region | Stock | Electric share of fleet |
| --- | --- | --- |
| China | 680,000+ | 30% |
| Europe | ~30,000 | ~2% |
| India | 11,500+ | <1% |
| United States | ~8,000 | <1% |
| Rest of world | ~50,000 | — |

Sources: IEA (International Energy Agency, 2025), Sustainable Bus (Sustainable Bus, 2025b)

## Sales trends

Global electric bus sales reached over 70,000 in 2024, a 30% increase year-on-year (International Energy Agency, 2025). China’s share of global sales has declined from ~99% in 2017 to less than 70% in 2024 as other markets grow, but it remains dominant.

Electric Bus Sales by Major Market

| Market | 2024 sales | Trend |
| --- | --- | --- |
| China | ~50,000+ | Rising (replacement cycle + scrappage scheme) |
| Europe (>8t) | 7,779 | +22% YoY; rose to 11,607 in 2025 (+48%) |
| India | 3,200+ | Overtook US in 2024 |
| South Korea | 2,800+ | Overtook US in 2024 |
| United States | ~2,000 | Declined ~40% from 2023 peak |
| Latin America | 2,000+ | Growing; Chile and Mexico leading |
| UK | ~2,000 | ~20% of European total |

Sources: IEA (International Energy Agency, 2025), Sustainable Bus (Sustainable Bus, 2025a)

# Electric Bus Share of New Sales

The share of electric buses in new sales varies enormously by market and bus type.

## China

Chinese city bus procurement is essentially 100% electric for new purchases (Sustainable Bus, 2024a). In H1 2024, 24% of all bus and coach sales (including intercity) were zero-emission, but the intercity coach segment was only 6% electric — long-range intercity routes remain harder to electrify.

Almost 70% of China’s 680,000+ electric buses were deployed before 2020 (International Energy Agency, 2025). Shenzhen fully electrified its fleet (~16,000 buses) by 2017. Much of the current demand is replacement-cycle driven, supported by a national city bus scrappage scheme announced in January 2025.

## Europe

EU city bus registrations reached 60% zero-emission in 2025, up from ~50% in 2024 and 35% in 2023 (Sustainable Bus, 2026c). Several northern European countries (Denmark, Finland, Netherlands, Norway) each exceeded 40% electric share in 2023–2024 (International Energy Agency, 2025).

Total European electric bus registrations (>8 tonnes) grew from 7,779 in 2024 to 11,607 in 2025, a 48% increase (Sustainable Bus, 2026a). Including other alternative powertrains, total non-diesel registrations reached 17,108 in 2025 (battery-electric 11,607; hybrid 2,565; CNG 2,378; fuel cell 558).

## India

India registered 3,644 electric buses in FY2024 (~4% of annual registrations), growing to 4,441 in calendar year 2025 (Sustainable Bus, 2025b, 2026b). The fleet has grown from fewer than 3,000 in 2020 to over 11,500 at end-2024. Tata Motors leads with ~40% domestic market share, followed by Olectra Greentech at ~18%. The PM eBus Sewa programme targets 10,000 buses across 169 cities.

## United States

The US electric bus market is surprisingly small relative to GDP. Sales declined ~40% in 2024 from the 2023 peak, partly due to Proterra’s 2023 bankruptcy and supply chain disruption (International Energy Agency, 2025). Electric school buses represent roughly half the US electric bus fleet, boosted by the $5 billion Clean School Bus Program (~8,100 buses funded). Federal funding of $1.7 billion has supported over 1,300 zero-emission transit buses.

# Manufacturer Market Shares

## Global picture

Chinese manufacturers dominate global electric bus production. BYD and Yutong are the two largest producers by volume. Yutong has delivered over 150,000 electric buses globally (cumulative). Over 15,000 electric buses were exported from China in 2024, more than 25% above the 2023 level (International Council on Clean Transportation, 2025).

## European market (most detailed data available)

The European market provides the most transparent manufacturer-level data. In 2025, the top nine manufacturers were (Sustainable Bus, 2026a):

European Electric Bus Registrations by Manufacturer (2025)

| Rank | Manufacturer | Origin | Units | Share | YoY growth |
| --- | --- | --- | --- | --- | --- |
| 1 | Yutong | China | 1,801 | 15.5% | +65% |
| 2 | MAN | Germany | 1,409 | 12.1% | +224% |
| 3 | Daimler/Mercedes | Germany | 1,395 | 12.0% | +52% |
| 4 | BYD | China | 1,305 | 11.2% | +206% |
| 5 | Iveco Bus | Italy | 1,091 | 9.4% | +33% |
| 6 | Wrightbus | UK | 921 | 7.9% | +7% |
| 7 | Solaris | Poland | 882 | 7.6% | +92% |
| 8 | Volvo Buses | Sweden | 364 | 3.1% | — |
| 9 | VDL | Netherlands | 353 | 3.0% | -12% |

Source: Sustainable Bus (Sustainable Bus, 2026a)

Chinese manufacturers (Yutong + BYD) collectively hold ~27% of the European market and are growing rapidly. However, European OEMs still hold the majority collectively. The top four manufacturers (Yutong, MAN, Daimler, BYD) control roughly 50% of the market between them.

For comparison, in 2024 the European rankings were (Sustainable Bus, 2025a):

European Electric Bus Registrations by Manufacturer (2024)

| Rank | Manufacturer | Units | Share |
| --- | --- | --- | --- |
| 1 | Yutong | 1,092 | 14.0% |
| 2 | Daimler/Mercedes | 918 | 11.8% |
| 3 | Wrightbus | 861 | 11.1% |
| 4 | Iveco Bus | 821 | 10.6% |
| 5 | Solaris | 460 | 5.9% |
| 6 | MAN | 435 | 5.6% |
| 7 | BYD | 426 | 5.5% |
| 8 | VDL | 401 | 5.2% |

Source: Sustainable Bus (Sustainable Bus, 2025a)

MAN’s jump from 435 units (#6) in 2024 to 1,409 (#2) in 2025 — a 224% increase — is notable, as is BYD’s tripling from 426 to 1,305.

# Battery Technology Evolution

## Battery capacity progression

Battery capacity for standard 12-metre electric buses has grown substantially over the past 15 years. The table below tracks concrete specifications from major manufacturers.

12m Electric Bus Battery Capacity by Model and Year

| Year | Manufacturer / Model | Battery (kWh) | Range | Chemistry |
| --- | --- | --- | --- | --- |
| 2010 | BYD K9 (1st gen) | 342 | 250 km | LFP |
| 2012 | Proterra EcoRide BE35 | ~54 | ~50 km | LTO |
| 2015 | Proterra Catalyst XR | 257 | 290 km | NMC |
| 2016 | Proterra Catalyst XR (upgraded) | 330 | 310 km | NMC |
| 2016 | Proterra Catalyst E2 | 440–660 | 310–560 km | NMC |
| 2022 | Proterra ZX5 Max | 738 | 480+ km | NMC |
| Pre-2024 | Yutong E12 | 344 | 350 km | LFP |
| 2024+ | Yutong E12 (upgraded) | 466 | 370 km | LFP |
| 2025 | Custom Denning Element 2 | 382–456 | 400–500 km | NMC |
| 2025 | Volvo BZL | 282–470 | up to 250 km | NCA |
| 2025 | MAN Lion’s City E 12 | ~480 | 550 km (test) | NMC |
| 2025 | New Flyer Xcelsior CHARGE NG (40ft) | 345–525 | 290–415 km | NMC |

Sources: BYD (EVMagz, 2024; Wikipedia, 2025a), Proterra (Green Car Congress, 2016; InsideEVs, 2022; Lambert, 2016; Wikipedia, 2025b), Yutong (Route One, 2024), New Flyer (New Flyer, 2025)

The trend is clear: battery capacity for a comparable 12m bus has roughly tripled to quadrupled over a decade. Yutong’s recent upgrade from 344 to 466 kWh on the same E12 platform — a 35% increase within a single generation — illustrates how rapidly the technology is improving.

Two different strategies are visible:

- BYD and Yutong use LFP (lithium iron phosphate) chemistry, which is cheaper per kWh and more thermally stable but heavier. BYD’s K9 has maintained ~342 kWh since 2010, focusing on cost reduction through manufacturing scale rather than capacity increases. Yutong has recently pushed LFP capacity higher.

- Proterra, Custom Denning, MAN, New Flyer use NMC (nickel manganese cobalt) chemistry, which has higher energy density (lighter packs for equivalent capacity) but at higher cost and with greater thermal management requirements.

## Battery cost trajectory

Battery pack costs have fallen roughly 90% over 15 years (Teslarati, 2024):

EV Battery Pack Cost (industry benchmark)

| Year | Cost ($/kWh) |
| --- | --- |
| 2008 | 1,415 |
| 2013 | ~600 |
| 2018 | ~180 |
| 2023 | ~139 |

Source: Based on BloombergNEF data compiled by Teslarati (Teslarati, 2024). Note: bus battery packs may be somewhat higher than the light-vehicle benchmark due to lower volumes and different specifications.

At $139/kWh, a 400 kWh bus battery pack costs roughly $55,000–60,000 — still a significant portion of the bus cost, but dramatically lower than a decade ago.

## Battery warranty progression

Yutong recently upgraded its battery warranty from standard terms to 10 years / 800,000 km at 70% state of health, with a 15-year / 1.5 million km option available (Route One, 2024). This reflects growing confidence in LFP cycle life and is relevant for fleet operators evaluating total cost of ownership.

# Real-World Energy Consumption

Energy consumption varies dramatically with conditions. For a standard 12m electric bus (Sustainable Bus, 2024b):

Electric Bus Energy Consumption (12m, real-world)

| Condition | Consumption (kWh/km) |
| --- | --- |
| Best case (20°C, light traffic, skilled driver) | 0.8 |
| Typical summer | 1.0–1.4 |
| Winter with diesel-assisted heating | ~1.5 |
| Winter with full electric heating | 2.3–2.5 |

Source: Sustainable Bus (Sustainable Bus, 2024b)

Cold weather (−5 to 0°C) can reduce range by up to 38%, primarily due to heating demand. Driver skill also matters: regenerative braking technique accounts for roughly 30% variation in consumption.

For Australian conditions (mild winters, hot summers with air conditioning), consumption of 1.0–1.3 kWh/km is a reasonable planning estimate. This implies a 400 kWh bus can cover 300–400 km per charge — well within the typical daily duty cycle for an urban route bus.

## Range records (test conditions)

Several manufacturers have demonstrated extended ranges under controlled conditions:

- MAN Lion’s City E 12: 550 km (24-hour TÜV SÜD monitored test)

- Iveco E-WAY: 527 km single charge (HVAC off)

- VDL Citea (490 kWh): 500+ km over 24 hours

These are not achievable in daily service but demonstrate the headroom in current battery technology.

# Key Observations

- China built early and big. Most of China’s 680,000 electric buses were deployed before 2020, driven by national subsidies. The city bus market is now essentially saturated — current demand is replacement-cycle.

- Europe is catching up fast. EU city bus registrations hit 60% zero-emission in 2025. The market doubled in two years (from ~6,400 in 2023 to ~11,600 in 2025).

- Chinese manufacturers are winning in export markets. Yutong leads the European market by volume (#1 in both 2024 and 2025). Combined with BYD, Chinese makers hold ~27% of European electric bus sales and are growing faster than European incumbents.

- The US is surprisingly behind. Proterra’s 2023 bankruptcy disrupted the market. India and South Korea both overtook the US in sales volume in 2024.

- Battery technology has matured. The 400–500 kWh range is now standard for a 12m bus, delivering 300–400 km of real-world range. This is sufficient for most urban route operations without midday charging. The constraint has shifted from battery capability to depot charging infrastructure.

- LFP vs NMC is the key chemistry divide. Chinese buses overwhelmingly use LFP (cheaper, safer, heavier). Western/Australian buses often use NMC (lighter, higher energy density, more expensive). Both approaches deliver workable products; the choice is primarily economic.

# References

EVMagz. (2024). BYD K9 electric bus: Specifications, battery and range. https://evmagz.com/byd-k9-electric-bus-specifications-battery-and-range/

Green Car Congress. (2016). Proterra boosts capacity of catalyst XR battery pack 28% to 330 kWh. https://www.greencarcongress.com/2016/05/20160516-proterra.html

InsideEVs. (2022). Proterra introduces 738 kWh battery packs for ZX5 buses. https://insideevs.com/news/580469/proterra-738kwh-battery-zx5-buses/

International Council on Clean Transportation. (2025). China’s zero-emission truck and bus market reaches historic high of 230,000 units sold in 2024. https://theicct.org/pr-chinas-zero-emission-truck-and-bus-market-reaches-historic-high-of-230000-units-sold-in-2024/

International Energy Agency. (2025). Trends in heavy-duty electric vehicles – global EV outlook 2025. https://www.iea.org/reports/global-ev-outlook-2025/trends-in-heavy-duty-electric-vehicles

Lambert, F. (2016). Proterra unveils new catalyst E2 all-electric bus with 350 miles of range on massive 660 kWh battery. https://electrek.co/2016/09/12/proterra-unveils-new-catalyst-e2-all-electric-bus-with-350-miles-of-range-on-massive-660-kwh-battery/

New Flyer. (2025). Xcelsior CHARGE NG. https://www.newflyer.com/bus/xcelsior-charge-ng/

Route One. (2024). Yutong electric bus battery capacities and warranties to increase. https://www.route-one.net/news/yutong-electric-bus-battery-capacities-and-warranties-to-increase/

Sustainable Bus. (2024a). China bus market 2024 first half. https://www.sustainable-bus.com/news/china-bus-market-2024-first-half/

Sustainable Bus. (2024b). Electric bus range and electricity consumption. https://www.sustainable-bus.com/news/electric-bus-range-electricity-consumption/

Sustainable Bus. (2025a). Electric bus market in Europe 2024. https://www.sustainable-bus.com/news/electric-bus-market-europe-2024-figures/

Sustainable Bus. (2025b). India bus market 2024 electric buses. https://www.sustainable-bus.com/news/india-bus-market-2024-electric-buses/

Sustainable Bus. (2026a). Electric bus market in Europe 2025. https://www.sustainable-bus.com/news/electric-bus-market-europe-2025/

Sustainable Bus. (2026b). India electric bus market 2025. https://www.sustainable-bus.com/news/india-2025-electric-bus-market/

Sustainable Bus. (2026c). Zero-emission buses reach 60% of EU city bus registrations in 2025. https://www.sustainable-bus.com/news/electric-buses-city-2025-europe-transport-environment/

Teslarati. (2024). EV battery prices dropped over 15 years. https://www.teslarati.com/ev-battery-prices-dropped-15-years/

Wikipedia. (2025a). BYD K series. https://en.wikipedia.org/wiki/BYD_K_series

Wikipedia. (2025b). Proterra catalyst. https://en.wikipedia.org/wiki/Proterra_Catalyst